Dear Above Board PS/PC User:

This diskette contains an alternate program for setting and

loading the Above Board PS/PC clock, ABCLOCK.

Above Board user Dr. E. Michael Perdue developed ABCLOCK and

placed it in the public domain.  ABCLOCK is useful in situations

where the Intel CLOCK.SYS device driver does not work.  For

example, ABCLOCK allows fractional second time increments from

the system clock.  Some graphics programs, keyboard utilities,

and data acquisition software requires fractional seconds.

USING ABCLOCK WITH THE INTEL ABOVE BOARD PS/PC:

1. Remove CLOCK.SYS from the config.sys file.

{ABCLOCK will not work if the Intel CLOCK.SYS device driver is

loaded.}

2. Copy the file ABCLOCK.COM from this diskette to your system

disk root directory.

3. Add the command ABCLOCK to your autoexec.bat file

SETTING THE DATE AND TIME WITH ABCLOCK:

Enter the following command from the DOS prompt

ABCLOCK /TIME=HH:MM:SS/DATE=MM-DD-YY

This sets the date and time on the Above Board clock.  It is also

possible to set only the time or date by using just one parameter

on the command line.

Because ABCLOCK was developed outside Intel, we do not cover it

with our product warranty.   Our thanks to Dr. Perdue for

creating this program and allowing us to distribute it.

ABCLOCK (c) 1986 by E. M. Perdue

Clock Utility for Intel Above Board PS/PC    All rights reserved

Dr. E. Michael Purdue

School of Geophysical Sciences

Georgia Institute of Technology,

Atlanta, GA  30332

#