Insert before "Printer Offset", p.6

______________________________________________________

Before changing the Printer Ready Flag make sure the Printer Port value

is correct for your system. Here's how;

With your printer On and ready to print, note the Flag value that is

shown following the `Printer Not Ready' message. This value, whatever

it is, should change as you turn the printer On/Off Line or turn the

printer power Off/On. If the value does change, as the printer status

changes, then your Printer Port value is correct. If the Flag value

remains the same then the Port selection must be changed, as described

in Section V, Ptr. Port (Dec).

#GPrinter Port#H

The computer's "address" that correspondes to the interface plug to

your printer is referred to as the Printer Port. While your system may

have more than one printer port, LABLMAKR only accesses the first

one which is also refered to as LPT1. Most system configurations use

the port addressed by (Decimal) 889, while some popular interface

boards use (Decimal) 957 as LPT1. If neither of these values are

correct for your system then refer to your printer interface

instructions for the proper value. Change the Ptr.Port (Dec) value as

described in Section V and repeat the Port test from the preceeding

paragraph. When you are satisfied that the Port value is correct then

the Flag value may be checked.

<6.1>

______________________________________________________

Insert before "Data Disk Drive #",p.8

_______________________________________________________

#E#GPtr. Port (Dec)#F#H - This entry allows the Printer Port value to

be re-defined when necessary, as described in Section IV. The typical

value is 889 but some system configurations may use a different

designation. It is important that the correct Port value be used for

the Printer Ready check. If the Port value is NOT CORRECT and the

Printer Ready Flag MATCHES THE TEST VALUE the flag indication is

ambiguous. In such a case, if the printer is NOT READY, the flag may

indicate (erroneously) that it is ready. Any attempt at printing will

result in an error condition which will dump the program and returns to

DOS.

<8.1>

______________________________________________________

______________________________________________________

#E#GPRINTERS#F#H

The IBM/Epson dot matrix is the Type 1 printer as defined in the

Re-Config screen. This printer type is representative of most printer

types used on compatible systems and will usually be the workable

selection.

Other selectable types are; 2 - Gemini 10 X, 3 - Tandy DMP 430,

and 4 - Epson 1500. These four types are representative of a wide

range of printer makes and types. If your printer is not included here

by name be sure to try the other type selections anyway. Generally one

of them will perform satisfactorily.

When selecting a different printer, from the Re-Config screen, the

program does not have to be reloaded. Just return to the Main Menu

either with e`X'it or `S'ave and the new printer parameters are active.

(If `S'ave is used this printer is selected when the program is

re-loaded.

#E#GTANDY Model 1000, 1200, 2000#F#H

These Tandy computers do not handle the LineFeed/Carriage Return

codes the same as other compatibles. If you are using one of these

computers you will have to modify slightly the initializing procedure.

Prior to Loading LABLMAKR you should Load the MS-DOS program, LF.COM.

This program is on the Tandy System diskette.

If you are using PC-DOS as your operating System the LF.COM file can be

transferred to it. This will allow an AUTOEXEC.BAT file to call for

LF.COM before calling LABLMAKR.

<6.2>

______________________________________________________

______________________________________________________

#E#GTab Labels - Printing#F#H

Label Types `O' and `P' are intended to be used for customized index

tabs. Such labels can be placed on the front side of standard die cut

index sheets. The normal format is `right-handed', the tab markings

being on the right edge of the label. On occasion a `left-handed' tab

label may be desired to place on the reverse side of the index sheet.

Provision is made to print either type.

The Re-Config screen offers a Tab Label control code, permitting an

Entry of `R', `L', or `R&L', corresponding to right/left handed or

both, when the label types `O' or `P' are printed. Although the entry

format for both types is right-handed they are printed according to

this control code. If the code does not include either an `L' or an `R'

the default is right-handed. When both types are selected the print

routine always prints the Right-hand version followed by the Left.

The type `P' tab label offers the 3 vertical fields for the tab edge.

Seldom are all 3 fields needed for tab information and the center and

outer-most fields are invariably used since they are most visible. When

the left-handed tab is printed these 3 fields are left-justified (a

blank field being ignored) which results in the best visibility of the

tab markings. This left-justification affects only the type `P' label.

<10.1>

___________________________________________________________