This is a 68000 debugger somewhat similar to DOS debug.  At present the

68000 is the only processor properly supported.

You invoke the debugger by a call to its entry point, dbg_start.  At

this time the debugger uses the values currently in the registers as

the working values.  The PC value is derived from the return address to

the entry point call.

The debugger may require to be run in supervisor mode.  I am unsure.

When initialized, it will hook the trace and illegal instruction

traps for internal use.  Other system resources are left alone except

for a small amount of scratch ram.

Debugger commands:

B - set/clr a breakpoint, show breakpoints

B		- show breakpoints.  Active breakpoints will be

preceded by a star.  There are 8 breakpoints;

however breakpoint 0 should not be used explicitly

as it is used implicitly by the 'G' and 'P'

instructions.

B+<num>,<addr>	- Set breakpoint # <num>

B-<num>		- Clear breakpoint # <num>

D - Dump memory

D		- Dump from current address (default = 0)

D<adr>		- Dump from new address and set current to next

available address.

D<adr>,<adr>	- Dump from first adress to second.  Set current.

E - Examine/change memory

E		- Examine from current address

E<adr>		- Examine starting at address

F - Fill memory

F<adr>,<len>,<val>	- Fills memory with value (byte wise fill)

G - Go /execute

G		- Run from current PC

G<adr>		- Run from <adR>

G,<adr>		- Run from PC and set a breakpoint at adr

G<adr>,<adr>	- Run from first adr and set a breakpoint at second

BDEFGPRTU

P - Proceed (step over)

P		- Step over the next instruction.  Subroutine calls

will be completely executed.

R - Show/Alter regs

R		- show regs

RD0		- Modify register D0.  Registers that may be modified

are:

D0-D7, A0-A7,SR,PC,USP

RD0,<val>	- Modify register d0 without a prompt

T - Trace (step into)

T		- Execute EXACTLY one instruction.  Subroutine calls

will terminate prior to the first instruction in the

called subroutine.

U - Disassemble

U		- Disassemble from current address

U<adr>		- Disassembler from <adr> and set a new current

address.