-------------------------------------------------------------------------------

This document was created to respond to the Frequently-Asked-Question:

What is the format of a Windows .WAV file?

That pops up in alt.sb.programmer and other newsgroups.

I have created a public domain (freeware) program that shows the basics of

reading a RIFF/WAVE file.  The program is waveinfo.exe and the source is

waveinfo.c.  Testing and support of this program is beyond the scope of this

project, period.

My main sources of information were Osborne's "Sound Blaster:  The Official

Book" first and second editions (which have the same mistakes) (do not bother

with either of these books).  And secondly the files RIFFMCI.TXT and MMREG.H

which I think I got from ftp.microsoft.com some time ago, I cannot find them

anywhere now which is why I included them.

Other sources for this sort of information:

FAQ: Audio File Formats (part 1 of 2)

The entire FAQ is also available by anonymous ftp from ftp.cwi.nl

[192.16.184.180], directory pub/audio, files AudioFormats.{part1,part2}.

The comp.compression FAQ has good stuff on compression.  This FAQ is

ftp'able from rtfm.mit.edu in directory

/pub/usenet/news.answers/compression-faq, files part1 and part2.

ftp.ncsa.uiuc.edu:misc/file.formats

Thanks to those who have not been named...

David Welch    dwelch@scf.nmsu.edu | dwelch@acca.nmsu.edu

-------------------------------------------------------------------------------

This is the 'typical' format of a Windows .WAV file

4 bytes "RIFF"

4 bytes basically the size of the WAVE chunk

4 bytes "WAVE" (we only care about WAVE chunks)

Any number of chunks (really sub-chunks) follow, a chunk is defined as:

4 byte identifier

4 bytes size

size number of bytes (chunk specific)

Any program must expect and ignore unknown chunks.

Two chunk types are required, the "fmt " chunk and the "data" chunk,

The "fmt " chunk must come before a "data" chunk

I am not sure if you can have multiple "fmt "+"data" chunks in the same file

The "fmt " (Format) Chunk:

WORD wFormatTag;              // Format category

0001h WAVE_FORMAT_PCM

etc

WORD wChannels;               // Number of channels

DWORD   dwSamplesPerSec;      //  Sampling rate

DWORD   dwAvgBytesPerSec;     // For buffer estimation

WORD wBlockAlign;             // Data block size

The "data" (Data) Chunk:

The WAVE_FORMAT_PCM (0001h) is the only format defined in the RIFFMCI.TXT

document and the only format I have seen to date (not that that means

much), so the following applies only to WAVE_FORMAT_PCM format tags.

wChannels=1 is mono, wChannels=2 is stereo, all other values are undefined

From the RIFFMCI.TXT file:

---cut---

The following diagrams show the data packing for a 8-bit mono and stereo

WAVE files:

Data Packing for 8-Bit Mono PCM:

Sample 1     Sample 2    Sample 3     Sample 4

Channel 0    Channel 0   Channel 0    Channel 0

Data Packing for 8-Bit Stereo PCM:

Sample 1                 Sample 2

Channel 0    Channel 1   Channel 0    Channel 0

(left)       (right)     (left)       (right)

The following diagrams show the data packing for 16-bit

mono and stereo WAVE files:

Data Packing for 16-Bit Mono PCM:

Sample 1                 Sample 2

Channel 0    Channel 0   Channel 0    Channel 0

low-order   high-order   low-order   high-order

byte         byte        byte         byte

Data Packing for 16-Bit Stereo PCM:

Sample 1

Channel 0    Channel 0   Channel 1    Channel 1

(left)       (left)      (right)      (right)

low-order   high-order   low-order   high-order

byte         byte        byte         byte

Data Format of the Samples

Each sample is contained in an integer i. The size of i is the smallest

number of bytes required to contain the specified sample size. The least

significant byte is stored first. The bits that represent the sample

amplitude are stored in the most significant bits of i, and the remaining

bits are set to zero.

For example, if the sample size (recorded in nBitsPerSample) is 12 bits,

then each sample is stored in a two-byte integer. The least significant

four bits of the first (least significant) byte is set to zero.Amplitude,

setting waveform

The data format and maximum and minimums values for PCM waveform samples of

various sizes are as follows:

Sample Size         Data Format         Maximum Value    Minimum Value

One to eight bits   Unsigned integer    255 (0xFF)       0

Nine or more bits   Signed integer i    Largest positive Most negative

value of i       value of i

For example, the maximum, minimum, and midpoint values

for 8-bit and 16-bit PCM waveform data are as follows:

Format       Maximum        Minimum Value    Midpoint Value                         Value

8-bit PCM    255 (0xFF)     0                128 (0x80)

16-bit PCM   32767 (0x7FFF) -32768 (0x8000)  0

---cut---