VGAFIX

Program to fix "invisible" VGA problem on 1000's

Jeffrey L. Hayes

When VGA is installed on a 1000-series that didn't come standard with VGA,

many programs don't see it and assume that CGA or EGA is being used.  This

is due to the failure of the Tandy BIOS to set the Video Configuration Code

byte at 40:8A for VGA.  There is also a problem with the BIOS on the

Tandy/Paradise VGA card, which returns the wrong information for Int 10h

function 1Ah.  Vgafixc and Vgafixm fix both problems.

To use one of these programs, put it in your Autoexec.bat.  These are not

TSR's, just little programs.  Use VGAFIXC if you have COLOR VGA.  Use

VGAFIXM if you have MONO VGA.  These programs will not work if you have EGA

or if you have a dual monitor system with VGA and something else (like

MDA); different (but similar) programs would be needed in that case.