Upgrading a 384k Racore/Quadram memory card to 512k for a total of 640k

by Roland Fritz & Hank Kennedy (Last update 01/14/90)

-----------------------------------------------------------------------------

Parts: 16  -  64k DRAMs  type 4164  (150 or 200 ns)

1  -  14 pin I.C. socket

2  -  30 ohm 1/4 watt resistors  5%

1  -  74HC08

2 ft of small gauge solid wire, also called wire-wrap wire

Note that the logic I.C.'s used may be either 74LS or 74HC.

Note that the board is removed from the computer for soldering.

It can be carefully lifted straight up from the expansion box.

Gently lift the right side (the side with the TMS4500 chip) to clear

the 60 pins from the bus.

This is not a job for those lacking good soldering skills.

This job is similar in work content to the instructions for upgrading

the Microsoft 128K sidecar to 512K.

Two to three hours labor and good solder skills are required.

1.---------------------------------------------------------------------------

Run memory diagnostics on the full 384k Racore/Quadram memory board

and troubleshoot until passing all tests.

Pull 2 banks of 8  64k DRAMs and set them aside (16 chips in two

adjacent rows--preferably banks 2/3 or 3/4)

There are 6 banks on the card with banks 0 & 1 soldered in.

Replace them with new 64k DRAMs and again verify all memory tests pass.

For each of the 16 DRAMs set aside and proven good earlier;

bend up pin4 a little above the top of the I.C..

Note that ceramic parts are extremely brittle where a leg joins the body.

Even with plastic parts try to bend the leg at several places in a U

shape to avoid breakage.

Straighten the other 15 pins on each DRAM to fit tightly over

another I.C. beneath it.

Take two rows of DRAMS out of their sockets and solder another DRAM piggy back

onto each one with pin 4 lifted on the top I.C.

Be careful to keep solder clear of the bottom portion

of the lower chip as it will be inserted back into a socket.

The chips must be aligned over the same number leads; that is pin one

over pin one etc..

Note that a small tip solder iron must be used and afterward use a lamp

to check the solder joints.

Insert the 16 piggy-back chips into the 16 sockets.

Solder a straight wire over the top of each row to connect all 8 pin 4's

of the piggy back I.C.s.

Solder a 30 ohm resistor to pin 4 at the end of each piggy back row

on the side nearest to the TMS4500A.

PAGE 1 OF 4

2.---------------------------------------------------------------------------

If you wish to check, the board should still boot up normally to 512k

total memory.

Lift the pin 4 leg out of its hole on the 74HC138  {G2a}

and wire it to pin 8 of that I.C.. {GND}

Lift pin 4 and pin 6 out of their holes on the 74HC368 for use later.

Unsolder the 74HC51 and remove it from the board,  if undamaged this

part can be used later.

WHILE THE 74HC51 IS OUT OF THE BOARD:

Shorten and bend up pins 2,3,5,6 and pins 8,9,10 on a new 74HC08 and mount

it on top of the 74HC08 that is next to the 74HC51 by soldering the legs

that were not bent.  The bent pins should not be touching anything.

Remove pin 6 and pins 8,9,10,11 from a new 14 pin I.C. socket and solder it

in place of the 74HC51 after the piggy back 74HC08 has been soldered in.

Install an 74HC51 into the new socket.

Wire HC51-6 to HC08-10 and then wire HC08-8 to the plated-thru hole at the

rear of the 74HC51 that HC51-6 used to go to.  That is the etch near pin7

of the 74HC51.  That etch is also connected to pin 4 of the TMS4500A.

If you wish to check, the board should still boot up normally to 512k

total memory.

3.---------------------------------------------------------------------------

Wire HC138-2 to HC368-4  {Add17}  and HC138-3 to HC368-6. {Add18}

Wire HC368-5 to HC51-10 and HC368-7 to HC51-9.

Wire HC368-2 to HC51-11  {Add19}  and HC51-8 to HC08-9.

If you wish to check, the board should still boot up normally to 512k

total memory.

4.---------------------------------------------------------------------------

Wire HC138-15 to HC08-2 and HC08-3 to the first piggy back 30ohm resistor.

Wire HC138-14 to HC08-5 and HC08-6 to the second piggy back 30ohm resistor.

The board should now boot up to 640k total memory.

Run memory diagnostics for the full 640k and verify that they pass.

5.---------------------------------------------------------------------------

o

My

PAGE 2 OF 4

RACORE/QUADRAM  384K BOARD

modified to 512k

___                                      --- existing etches

___ ,-4-|   \

CAS-|   | 08 )-6--all rows,pin15                  x-- I.C. pin lifted

`-5-|___/

,-----------------------.      *** new wiring

|    ___                |

these four gates    `-1-|   \               |      -+- junction

are on the I.C.       | 08 )-3--row2,pin4 |

next to the      ,--2-|___/               |       o- inverted logic pin

74HCT245         |          74HC08        |

| ,----------------------(--.

| |   ___                |  |    all etches to pin4 of a

___      | `9-|   \               |  |      row of DRAMs go through

____ ,-12|   \     |    | 08 )-8--row1,pin4 |  |      a 30 ohm resistor

RAS1-|   | 08 )-11-+--10|___/               |  |

`-13|___/     |                        |  |

`------------------------(--(--. these two gates are on the

|  |  |   piggy back 74HC08

|  |  |        ___

a,b,c select 0-7              |  |  +*****1-|   \     piggy row1

g1 high and g2a&b low to enable          |  |  |       | 08 )-3*/\/\-*,pin4

,****(**(**(*****2-|___/   30ohm

{Add16}  B7--------.  74HC138     *    |  |  |        ___

____1_____         *    |  |  +*****4-|   \     piggy row2

{Add17}         |    A     |        *    |  |  |       | 08 )-6*/\/\-*,pin4

B8----------------2| B     Y0 |o-15****' ,**(**(**(*****5-|___/   30ohm

{Add18}    |          |          *  |  |  |

A7-------------3| C     Y1 |o-14******'  |  |  | the four gates below are

GND    |          |             |  |  |   next to the 74HC51

B9--x  `**4-o| G2a   Y2 |o-13---------(--'  |        ___

{Add19}    ____    |          |             |     +-----1-|   \

RAS0-5-o| G2b   Y3 |o-12---------'     |       | 08 )-3---row3,pin4

|          |               ,---(-----2-|___/

+5v--6| G1    Y4 |o-11-----------'   |        ___

|          |                   +-----4-|   \

,7-o| Y7    Y5 |o-10---------.     |       | 08 )-6---row4,pin4

|   |    Y6    |             `-----(-----5-|___/

|   `----o-----'                   |        ___   74HC08

|        |                         +----10-|   \

|        9                         |       | 08 )-8---row5,pin4

|        `-------------------------(-----9-|___/

| 74HC368                          |        ___

`---------------------------.      `----12-|   \

|              | 08 )-11--row6,pin4

`-----------13-|___/

PAGE 3 OF 4

{Add19}

|\           ___

B9--+----2-| >o-3-+--3-|   \

*      |/     |    | 51 )-, ____

A7-+---(-------------(--2-|___/  `-\   \

*   *  {Add18}    |     ___      |51 >o-6-.  x-----.    TMS4500, pin4

*   *             `--5-|   \  ,-/___/     *        |        ______

*   *                  | 51 )-'           *        `-----*--MEMSEL

B8+---(---(----------------4-|___/              *     ___      *

*   *   *  {Add17}                            `*10-|   \     *

*   *   *                    74HC51                | 08 )-8-*'

*   *   *                   ___               ,*-9-|___/

*   *   *         GND-+--1-|   \              *

*   *   *             +-13-| 51 )-, ____      *

*   *   *             `-12-|___/  `-\   \     *

*   *   *                   ___      |51 >o-8-'

*   *   `**************-11-|   \  ,-/___/

*   *       |\     ,***--9-| 51 )-'

*   `****-6-| >o-7-' ,*-10-|___/

*           |/       *

*       74HC368      *               x--. disconnected from the HC51-9,10,11

*           |\       *               x--+   and the HC368-4,6

`********-4-| >o-5-**'               x--+

|/                   x------+

x------+---GND

PAGE 4 OF 4

******************************************************************

Additional Tip for 3.5" drives using Racore diskette controler bd:

To minimize disk errors when using 3.5" drives, locate and replace

the R11 resistor on the diskette card. Change from a 390 ohm

(orange,white,brown) to a 750 ohm (violet,green,brown) using a 1/4

watt resistor. I believe that this board was only required if your

Racore had the DMA option.

On newer Racor units the extender board that connects its memory

board to the sidecar has a solid copper ground plane over the bottom

surface. My system didn't and as a result, there was electrical

interference with the lower floppy disk drive, even before upgrading

the memory. The fix is to install a thin metal shield or plate under

your extender board and then solder a wire from it to a known good

ground. A folded sheet of aluminum foil connected to a ground clip.