NEAT Expanded Memory Driver 4.0 Version 1.52

NEAT EMS 4.0 DOCUMENTATION.

The EMS (Expanded Memory Specification) Version 4.0 device

driver is a standart MS DOS device driver that gets loaded at boot

time by the  CONFIG.SYS file. Because  EMS 4.0  is a device driver,

is should look like this in the CONFIG.SYS file.

DEVICE=EMM.SYS

With  no  anything else  included in this command line,the

EMS driver will get loaded with the default hardware values of the

I/O  address that enables the  EMS and the memory address used for

EMS  paging. These default values are dependent on the NEAT imple-

mentation of EMS.

To  change  these  default values, as well as to add a few

extra features, the  EMS  device driver command line can look like

this:

DEVISE=EMM.SYS -Ix -My -Pzz -D

Where:

I  = I/O adress to enable EMS page

x = 0 - 208h

= 1 - 218h

= 5 - 258h

= 6 - 268h

= A - 2A8h

= B - 2B8h

= E - 2E8h

M  = Page frame address (address of 64K window)

y = 1 - C000h

= 2 - C400h

= 3 - C800h

= 4 - CC00h

= 5 - D000h

P  = Maximum number of open handles (default to 64)

zz= number of handles (1-128)

D  = Enable EMS diagnostics

The  command line parameters can  be in  either  upper  or

lower case.

When the EMS driver is first executed, a title and copyright

notice will appear on the screen as follows:

"EMS Expanded Memory Device Driver Ver. 4.0"

"Copyright (c) Chips and Tecnologies Inc., 1987"

If the EMS driver detect that a prevous EMS driver has been

loaded,it will put this message on the screen:

"An Expanded Memory Manager has already been installed."

The EMS  driver will then not get loaded into the system.

When the  EMS  driver gets loaded by  MS DOS, an internal

check will  be done on the EMS  configuration in hardware. If the

driver detect any conflicts that would cause EMS not function pro-

perly, the message:

"The EMS setup has been incorrectly specified. No EMS is available."

will appear  on the screen after the EMS copyright notice and  EMS

will be disabled. If the  EMS  hardware is configured properly and

the  I/O  address is overriden  by the command line parameter, the

screen will show this message:

"The EMS I/O adress has been changed."

If the EMS page frame adress is being overriden  from the

command line,  the  following message  will appear on the screen:

"The EMS Frame Segment has been changed."

If the user specifies an invalid parameter(s), the default

values, set up by the BIOS or the setup program, will be used.

If the EMS diagnostics enabled from the command line, the

following message will appear on the screen:

"Testing EMS Expanded Memory Page Number:NNN."

The NNN represents the page number currently under test.

If the diagnostics fail, the EMS will be disabled and the

following message appears on the screen.

" Expanded Memory FAILED diagnostics test. "

If the diagnostics pass, or if diagnostics were not

specified on the command line, the EMS driver has been loaded

correctly and EMS is enabled. The final message to appear on the

screen is:

"There are XXX pages, or YYY Kbytes of EMS Expanded Memory on the

system."

The System can now be used to support

Lotus/Intel/Microsoft LIM EMS 4.0 in the same manner as standart

EMS cards.