Some languages and non-IBM versions of BASIC do not allow you to use LPT3: for

the output to the SPEAK.COM program.  If you get a "device unavailable error"

or your output goes to your printer when you try to "speak" to LPT3: there are

other versions of the SPEAK.COM program on the disk.

These are labeled SPKLPT1.COM, SPKLPT2.COM, SPKLPT3.COM, SPKCOM1.COM, and

SPKCOM2.COM.  These versions work with LPT1:, LPT2:, LPT3:, COM1:, and COM2:.

To use one of these versions, first copy the file over to SPEAK.COM.  Then

re-boot your computer and use the new version of SPEAK.COM. From then on use

the device you selected whenever you would normally use LPT3:.

An alternative when using Turbo Pascal with LPT3: is use code similar to that

shown below:

{//////////////////////////////////////////////////////////

//         This program demonstrates how to use          //

//         the "SPEAK" program of the ECHO PC2           //

//         from Turbo Pascal.  The program uses          //

//         a small amount of inline assembly code        //

//         to redirect the "USR" output to "LPT3".       //

//////////////////////////////////////////////////////////}

program speak;

var

phrase: string [255];

procedure LPT3;

begin

inline($58/           { POP   AX    }

$5A/           { POP   DX    }

$50/           { PUSH  AX    }

$8B/$C2/       { MOV   AX,DX }

$B4/$00/       { MOV   AH,00 }

$BA/$02/$00/   { MOV   DX,02 }

$55/           { PUSH  BP    }

$CD/$17/       { INT   $17   }

$5D/           { POP   BP    }

$C3);          { RET         }

end;

begin

UsrOutPtr := ofs(LPT3) + 04;

write ('Enter phrase: ');

readln(phrase);

writeln(Usr, phrase);

end.

#