**AWINTAB.DOC file 12/12/95

Using the Tablet with Windows 3.x and Windows 95

================================================

Your tablet really brings the Windows 3.x and Windows 95 to life by

providing the ultimate control of absolute positioning and a wide range

of customization options. With these features, your tablet lends even

greater ease of use to your Microsoft Windows desktop and applications,

letting you work the way you think. The tablet's absolute positioning

capability and customization options are designed to boost your

performance, helping you work easily, quickly, and intuitively. The

WinTab driver also lets the tablet work under relative positioning like

a mouse, if necessary, though you will find this mode is less efficient.

The WinTab driver allows you to use the tablet only or to coexist

with another pointing device such as a mouse or trackball with your

Windows system. You can drive the system pointer from the tablet and

another pointing device such as a mouse on the fly.

When you install the powerful WinTab driver under Windows system, all of

your Windows applications are taken care of at the same time. You do not

have to install or setup the tablet for each application. The WinTab

driver offers full Windows system compatibility.

If your Windows application also supports a WinTab compatible tablet,

you can configure the digitizer options of your Windows application

as a WinTab compatible tablet. With these applications, you can use

the tablet as both a Windows system pointing device for all Windows

applications and as the primary pointing device for template picks for

applications such as AutoCAD for Windows.

In addition to installing the tablet under Windows system, you may also

want to use the Tablet Control Panel program - an application that lets

you customize the use of your tablet. For increased comfort and ease of

use, you can use the Tablet Control Panel to adjust the size of the

tracking area on the tablet. For example, you can set a 2"x2" area on

the tablet to correspond to the entire Windows screen, making menu

picking as easy as a flick of the wrist. You may also use the Tablet

Control Panel to customize the button assignments and increase or

decrease the speed and sensitivity of your tablet's pointing device.

With the Tablet Control Panel, you can also select the Aspect Ratio

function to ensure perfect one to one synchronization between your

tablet and monitor screen. This function is especially important for

graphics applications where precise rendering is required.

For the details about the functions of the Tablet Control Panel,

please refer to the section - Using the Tablet Control Panel.

Note    If you are planning to upgrade to Windows 95 from Windows 3.x,

we recommend you use our easy Auto-Install program to install

the Wintab driver and Tablet Control Panel for your Windows 3.x.

Next, when you upgrade your system to Windows 95, the Windows 95

setup program will do the necessary things automatically.

Installing the Wintab Driver & Tablet Control Panel for Windows 3.x

-------------------------------------------------------------------

To install the Wintab driver and Tablet Control Panel, we recommend

to use our easy Auto-Install program which will install the Wintab

driver and Tablet Control Panel automatically for your Windows 3.x.

You can choose to run Windows with your tablet only or to use the tablet

coexist with another pointing device such as a mouse or trackball

through the installation procedure.

To install the WinTab driver for Windows 3.x:

1. Insert the Drivers/Utilities diskette in your floppy drive.

2. At the DOS prompt(if you are in Windows, exit Windows), type A:

(where A = the designation for your floppy drive) and press <ENTER>,

then type INSTALL and press <ENTER>. First answer the on screen

question concerning the COM port number that you connected the tablet

to. When the menu is shown on the screen, please select the item

"Install Windows driver for Windows 3.x" and follow the directions

on the screen.

3. After the installation is complete,press <ESC> to quit the program.

Now you can use the ACECAD tablet to run Windows 3.x.The tracking area

is the whole active area (12"x12" for A-1212 or 18"x12" for A-1812) of

the tablet. I.E. the whole active area of the tablet corresponds to the

whole Windows screen. The Easy Auto-Install program creates a group

named ACECAD in the Program Manager window. Open the ACECAD group. You

will find a document file named awintab.doc and a Tablet Control Panel

program.

You can use the Tablet Control Panel to customize the use of the tablet.

Please refer to the following section - Using the Tablet Control Panel.

Installing the Wintab Driver & Tablet Control Panel for Windows 95

------------------------------------------------------------------

To install the WinTab driver for your Windows 95, please complete the

following steps:

1.      Choose the Start button on the lower-right corner to pop up

the start menu and choose the Settings item.

2.      The sub-menu of Settings appears then choose the Control Panel.

The Control Panel Window appears.

3.      Choose the Add New Hardware icon. The Add New Hardware Wizard

dialog box appears. Then click the Next button.

4.      Highlight No radio button to disable auto-detection of hardware,

then click the Next button.

5.      In the List of hardware types, highlight the Other devices item

and then click the Next button.

6.      The lists of Manufacturers and models of Other devices appear and

click the Have Disk button.

7.      The Install From Disk dialog box appears, prompting you for the

location of the WinTab driver by displaying a highlight bar with

A:\. Put the Drivers/Utilities diskette in your floppy drive. You

will need to change the path name to A:\AWINTAB (where A= the

designation for your floppy drive) and click the OK button.

8.      The driver "ACECAD Tablet (WinTab Driver Vx.xx)" will be listed in

the box. Then click the Next button.

9.      The Add New Hardware Wizard dialog box appears again. Click the

Finish button to finish the installation.

10.     The Transducer Setup dialog box appears. Select the type of pointing

device (cursor-puck or stylus-pen) which you have connected to the

tablet. Click OK button.

11.     The System Settings Change dialog box appears. Click Yes button

to restart your computer. Now you can run your Windows 95 with the

tablet only or your tablet can coexist with another pointing device

such as a mouse or trackball. The Tablet Control Panel named ACECAD

Tablet will be added to the Control Panel of Windows 95.

You can use the Tablet Control Panel to customize your tablet settings

such as the Tracking Area, Button Assignments, etc. for even greater ease

of use.

Note    If you run Windows 95 with the ACECAD tablet and another

pointing device such as a mouse or trackball, the mouse

operation in the DOS prompt full screen session under Windows 95

is provide by the other pointing device then the ACECAD tablet.

If you need the mouse operation in the DOS prompt full screen

session under Windows 95, you have to load the ATABLET.COM after

you create a new DOS prompt session. For the details of loading

the ATABLET mouse driver for DOS, please refer to the ATABLET.DOC

in the ATABLET directory of this diskette.

Using the Tablet Control Panel

------------------------------

The Tablet Control Panel allows you to adjust the tracking area on

your tablet and make other customization adjustments for optimum

performance in Windows. Open the Tablet Control Panel from the opened

group window named "ACECAD"(created by the INSTALL.EXE program) in

Windows 3.x or simply double click the ACECAD Tablet icon in the Control

Panel window of Windows 95. The Tablet Control Panel Window will

display three icons - Information, Features, and Customization.

The following list summarizes the options you can set.

*   Choosing the Information icon, you can see the background

information for the tablet and change the tablet's unit of

measure from standard to metric.

*   Choosing the Transducers icon, you can select the exact type of

pointing device you will use. This selection assigns the

default button configurations for each pointing device.

*   Choosing the Customization icon,  you can change the following

options in the Tablet Setup dialog box;

-  Tracking Mode

-  Button Assignments

-  Tracking Area

-  Aspect Ratio

Tracking Mode (Relative or Absolute Mode)

*****************************************

If needed for a specific application, you can change the Tracking

mode from absolute mode to relative mode. In Absolute mode, you can

adjust the tracking area. By reducing the size of the tracking area,

while in Absolute mode, it will also increase the speed of your

pointer movements.

To adjust the tracking of the screen pointer for Relative mode,

change the percentage for the sensitivity parameter. This percentage

is the ratio of the scaled resolution of motion to the resolution of

the tablet. For example, the resolution of the tablet is set to 500

lines per inch (LPI) and the sensitivity is 40%. This means that the

tablet has the same resolution as a 200 LPI (500 LPI x 40%) mouse.

Increasing or decreasing the Sensitivity percentage will increase or

decrease the response of the screen pointer.

Note!   The sensitivity of the tablet's pointing device is set

through the Tablet Control Panel application, not through the

Mouse icon in the Control Panel of Windows.

Button Assignments

******************

When the Default option is selected, from the drop-down list box in

this Button Assignments option, you can see the default button

assignments for the tablet's pointing device you choose in Transducer

Setup dialogue box. You also can use this option to redefine the

button assignments such as left click, left double click, left drag,

right click, ...and so on.

Tracking  Area

**************

The default tracking area is the whole active area (12"x12" for A-1212,

18"x12" for A-1812) contained by the recessed area covered by the

transparent overlay sheet. Under absolute mode, the position of the

tablet's pointing device on the tablet corresponds directly to the

screen position. If you are using the tablet primarily for menu picking

and pointer movement, you may wish to reduce the size of the tracking

area. With a smaller tracking area, you can move from one corner of the

Windows screen to the other within a short distance on the tablet, thus

substantially increasing your speed and ease of operation.

Changing the Tablet's Unit of Measure

*************************************

The unit of measure for the tablet can be set to inches (") or

centimeters (cm). To change the unit of measure:

1.  Choose the Information icon from the Tablet Control Panel window.

Or

Choose the Customization icon. Both the Tablet Setup window and

the Tablet Information window let you set the unit of measure.

2.  Open the Units drop-down list box and select inches or

centimeters.

3.  Choose the OK button to accept the change.

Changing the Tablet's Pointing Device

*************************************

You must select the proper type of pointing device which you have

connected with the tablet for proper default button assignments. To

change the type of pointing device:

1.  Choose the Transducers icon from the Tablet Control Panel window.

2.  Open the Cursor Type drop-down list box and select the tablet's

pointing device you use.

3.  Choose the OK button to accept the change.

Changing the Tracking Mode and Sensitivity Parameters

*****************************************************

To change the tracking mode:

1.  Choose the Customization icon from the Tablet Control Panel

window.

2.  In the Tablet Setup window, click the appropriate option button

to select Relative or Absolute mode.

If you select Absolute mode, you can reduce the size of your tracking

area to increase the speed of your pointer movements. Using a small

tracking area will make the pointer very sensitive to your hand

movements.

If you select Relative mode, adjust the sensitivity through the

sensitivity edit box:

1.  Enter a number from 1 to 100 in the edit box next to Sensitivity.

Or

Click the up or down arrow to change the number.

Changing the Button Assignments

*******************************

You can select the default button settings or change the button

assignments to suit your needs. To modify the default settings:

1.  Clear the Default check box.

2.  Select an option from the drop-down list box.

3.  Choose the OK button to accept the new setting.

Changing the Tracking Area

**************************

While working in Absolute mode, you may need to adjust the size and

location of the tracking area. While working in Relative mode, we

recommend you use the whole active area as the tracking area. To

specify a new tracking area:

1.  Clear the Whole Tablet check box.

2.  Click the active area rectangle in the lower-middle section of

the Tablet Setup dialogue box.

The screen pointer changes from an arrow to a cross.

Use the stylus-pen tip button (or the #1 button on the cursor-

puck) to click the lower-left corner and then the upper-right

corner of the new tracking area you want to use.

Or

Type the coordinates into the text box in the lower-left corner

of the Tablet Setup dialogue box.

Or

Click the up or down arrows next to each of the coordinates to

adjust the size of the area.

3.  If you need to preserve the aspect ratio of the new tracking

area to the area on your screen, select the Preserve Aspect

Ratio check box.

Note!   For tracing or precise drawing, you will probably want to

preserve the aspect ratio. For selecting menu options or

simple pointer movements, this setting is not required.

4.  Choose the OK button.

The tracking area you specified for the tablet now corresponds to the

your entire Windows screen.  Your tablet's pointing device will now

respond only within the new tracking area on the tablet, not the full

area covered by the overlay.