17 April 1998

This file contains the most up-to-date information on the following topics:

1.Important Backup Information

2.Caution—Adapter Use

3.Precautions

4.Using a Pointing Device During Rebooting and Resuming

5.Using Infrared Communications

6.Avoiding Infrared Printing Problems

7.Avoiding Problems with Infrared Operations

8.Restoring Proper Infrared Operation

9.Microsoft's "Infrared Transfer" Application (Windows 95)

10.Do Not Enable "Autodetect CD"

11.Synchronizing an External Display or Projector

12.Using Older Models of Microsoft Mice

13.Cannot Use ESC during Boot Process

14.Installation of Microsoft Internet Explorer 4.01

15.Taskbar Mute / Unmute indicator can be incorrect after suspending to disk

16.Occasional Difficulty Inserting PC Cards and Floppy disks

17.Default BIOS setting for system resources

18.If you need to reinstall Phoenix PowerPanel

19.Avoid Standby and Suspend setting

20.Universal Serial Bus (USB) Disabled on your OmniBook

As your primary sources of information, use the following:

The Introducing Microsoft Windows 95 manual: This is shipped with your OmniBook and

contains information about using the standard features of Windows 95.

The OmniBook website at http://www.hp.com/omnibook: The Corporate Evaluator's Guide at

our website provides complete information about installing alternate operating systems, as

well as how to configure the OmniBook in a corporate, networked environment.

----------------------------------------

1. Important Backup Information

Because the HP OmniBook Sojourn is repaired by exchange only, you should regularly back up your data.  To minimize your inconvenience or loss of data files, you should back up your data files regularly!  Backup files are important because:

- If you accidentally delete or lose a file on your hard disk, a backup copy provides a convenient recovery method.

- The HP OmniBook Sojourn is repaired by exchange only—you do not get your unit or hard drive back as part of a service exchange.  Service exchange units have the operating system preloaded on the hard drive and are ready for you to load your applications and data files.

----------

2. Caution—Adapter Use

Use the HP OmniBook Sojourn ac adapter (F1434A) and auto/airline adapter (F1435A) with HPOmniBook Sojourn products only. The voltage and polarity of these adapters can damage other equipment.

----------

3. Precautions

This section describes certain situations that could cause serious loss of data.

Do not remove a data storage PC Card while it is actively reading or writing--its data could become corrupted.

Certain applications, such as Intuit's Quicken, keep their working files open, and they update the files as you make changes. For such applications, you should open only files that reside on the hard drive, not on a removable disk.  An open file on a removable disk (such as a floppy disk or PC Card) is susceptible to corruption in certain situations.

----------

4. Using a Pointing Device During Rebooting and Resuming

While the OmniBook is rebooting, resuming (turning itself on after suspension), starting Windows, or exiting Windows, do not touch the touch pad or an attached external pointing device.  This may result in the pointing device not functioning. Wait until the screen is stabilized and the hour glass ceases to appear.  If the pointing device is not functional, try the following steps, in this order, until it is functional. While performing these steps, do not touch the external pointing device.

1.Press any key on the keyboard.

2.Suspend the OmniBook and then resume it (press the power button briefly to suspend, then

press the button again to resume).

3.Shut down Windows, then turn the OmniBook off and on (Start menu, Restart).

----------

5. Using Infrared Communications

Infrared communications are, by default, not enabled on the OmniBook computer. To use infrared communications, read the readme files and and load the drivers supplied on the hard drive or on the Recovery CD-ROM under \OmniBook\Drivers. Look in the subdirectory for your operating system, then look under Fast IR or IrDA.

----------

6. Avoiding Infrared Printing Problems

If you have problems with infrared printing or problems while infrared printing is enabled, check these suggestions:

Try setting up the infrared printer as COM4 instead of LPT3.  COM4 is an alternative infrared

port.

DeskJet 340, 690, 820, 850, and 870 printers must use COM4. For the DeskJet 340 in Windows

95 you must also limit the connection speed (Options tab in the IR Monitor) to 38.4kbps.

----------

7. Avoiding Problems with Infrared Operations

To avoid problems with infrared communications or printing, close applications using IR or any folders opened remotely over an IR link and disable IR in the IR Monitor before suspending the OmniBook or docking.

----------

8. Restoring Proper Infrared Operation

Sometimes the infrared software gets into an error state so that further infrared operations do not work.  Try one of the following suggestions to restore proper operation:

In Windows 95 disable and re-enable infrared communication using the IR Monitor Options tab. Be sure to click on "Apply" or "OK" after disabling and after re-enabling.

Close all applications and reboot the OmniBook.

----------

9. Microsoft's "Infrared Transfer" Application (Windows 95 Only)

To avoid problems with Microsoft's Infrared Transfer application, enable it only when you actually need to use it and then disable it again.  To enable or disable Infrared Transfer, right click on the Infrared Recipient icon in My Computer and click on the Enable/Disable Infrared transfer menu entry.

----------

10. Do Not Enable "Autodetect CD"

If "Autodetect CD" is enabled on your OmniBook and a CD-ROM is present, your OmniBook will not automatically turn off after a period of inactivity. For this reason, "Autodetect CD" should not be enabled on your OmniBook.

----------

11. Synchronizing an External Display or Projector

If you connect an external display, projector, or projection panel to the

OmniBook display output, and its image is not stable, try one of the following:

- Use a display or projector that matches the resolution of the OmniBook display (SVGA).

-or-

1.Change the Display Properties of the OmniBook to match the resolution of the projector or external display: In Control Panel, Display, Settings, change the Desktop Area to 800 by 600 pixels or to 640 by 480 pixels.

2.Turn off the OmniBook's display and retain the external display by pressing SHIFT+ALT+BACKSPACE, as needed.

----------

12. Using Older Models of Microsoft Mice

The OmniBook cannot use Suspend mode with two of Microsoft's older mice models (parts #28898 and #07685). The suspend and resume activity causes the OmniBook to lock up with these mice. If you are using one of these mice, do the following to prevent this problem:

1. Open the BIOS Setup utility (reboot, then press F2).

2. In the Power menu, disable the setting for Resume on Modem Ring.

3. Exit the Setup utility.

----------

13. Cannot Use ESC during Boot Process

Contrary to the information in the User's Guide, chapter 6 (Troubleshooting), under Accessory Problems, "If you can't boot from a CD in the CD-ROM drive":

You cannot press ESC to interrupt the boot process and make temporary changes.

----------

14. Installation of Microsoft Internet Explorer 4.01

Microsoft Internet Explorer 4.01 was installed when you started your OmniBook for the first time.

However, only the minimum required Microsoft Internet Explorer 4.0 files have been installed, including the Internet Explorer 4.01 browser and Outlook Express.

If you want to uninstall Microsoft Internet Explorer 4.01, you can do so by going to the Control Panel / Add/Remove Programs and selecting Microsoft Internet Explorer 4.01 in the Install/Uninstall tab.

If you wish to have a complete installation of Microsoft Internet Explorer 4.01 on your OmniBook, including all applets and tools, run the "Internet Explorer 4.0 Active Setup" located under Start Menu / Programs / Accessories / Internet Tools.  This installation will allow you to install one or more of the following applets and tools:

* Netmeeting

* FrontPage Express

* NetShow

* Web Publishing

* Chat

* Multimedia Enhancement including Real Audio

----------

15. Taskbar Mute / Unmute indicator can be incorrect after suspending to disk

Pressing the F10 function key mutes and unmutes your audio.  However, after resuming from Suspend to Disk, the mute / umnute indicator on the Taskbar will not immediately reflect this action. The indicator will correctly indicate the mute / unmute status after your audio hardware is accessed, such as when you begin playing an audio CD.

----------

16. Occasional Difficulty Inserting PC Cards and Floppy disks

Infrequently, you may experience some difficulty inserting PC Cards into the OmniBook PC Card slots.  This is especially true if there are any rough or raised areas on your PC Card.  Remove and reinsert the PC Card and/or jiggle the PC Card slightly when inserting.  This difficulty may also occur occasionally when inserting a Floppy disk into the OmniBook Media Slice Floppy drive.

----------

17. Default BIOS setting for system resources

The default OmniBook BIOS setting is to have system resources (IRQs, DMA, etc.) handled by Windows 95.  If you need to configure these devices manually (such as when running Guardian or operating your OmniBook in DOS mode) switch the 'OS Legacy Mode Support' BIOS setting to 'No'.

----------

18. If you need to reinstall Phoenix PowerPanel

Your HP OmniBook Sojourn comes equipped with the Phoenix PowerPanel software. We recommend you use that software on your HP OmniBook Sojourn.

If you should ever need to reinstall the PowerPanel application, just run the SETUP.EXE

program located in the C:\OmniBook\Drivers\Win95\PPanel\Disk1\ directory and follow the instructions.

Phoenix PowerPanel is an application that gives users complete control over the power management behavior of their systems as well as key information about system activity and

battery life.

PowerPanel

- extends system battery life by allowing easy change of power management behavior with a

single icon click.

- provides method for direct power control of devices, such as video, hard-drive, PC Card, sound

system or other devices.

- drag and drop other applications and program icons from the desktop to the PowerPanel

toolbar.

- replaces character-based SETUP or Pop-Up power management screen with simple and

powerful icon-based Application Bar.

You can find more information about Phoenix PowerPanel at the following URL:

http://www.phoenix.com/notebook/ppanel.html

----------

19. Avoid Standby and Suspend setting

By default, your OmniBook is configured to disallow going directly from Standby power mode to Suspend mode (Suspend-to-RAM or Save-to-Disk) because doing so may result in poor system performance or a system crash.  Your PowerPanel profile and BIOS settings should not be modified to allow Standby to Suspend.

----------

20. Universal Serial Bus (USB) Disabled on your OmniBook

Universal Serial Bus (USB) is a recent and evolving technology.

By default, USB is disabled on your HP OmniBook Sojourn.

To gain a good understanding of current limitations and how USB

will perform on Sojourn, please consult the OmniBook website

at http://www.hp.com/omnibook.

Click on Support followed by Software/FAQs.

Then look under HP OmniBook Sojourn PC.  You will find USB

drivers and a review of USB issues to enable the best possible

implementation of today's USB features on your Sojourn.

- End of File -