Using the MS-DOS 2.0 small-c Compiler

I.      General Comments

The  MS-DOS  2.0 small-c compiler requires an H-100/150 series system with

128K  of  RAM  and  two TM100-2 style (DS 40 track) drives. The compiler is for

MS-DOS  version  2.X  and  will  not  work on earlier versions of MS-DOS (e.g.:

ZDOS).  The  compiler  will  be  able to work with more complicated programs if

enough  memory is available for the stack; the transient program area should be

a  full  64K  long.  Additional  disks  (or  higher-capacity  drives)  are  not

mandatory,  but  will  be  helpful.  This  document  will  assume  the  minimum

configuration given above.

Because of the size of the compiler and the runtime support library, it is

not  possible  to use a single 5-1/4" disk for both compiling and assembling if

the program being worked on is large. Therefore, to use the compiler you should

first make a bootable disk. On this bootable disk (disk ALPHA), first build the

directory  "\C"  and  then the subdirectory "\C\INCLUDE". This is the directory

where  C88 expects to find include files by default. In this directory, put the

file  STDIO.H.  Elsewhere  (e.g.: "\"), put C88.COM, your favorite text editor,

CRUN.OBJ,  the runtime support library, the post-compilation script, CMAKE.BAT,

the  small-c  utilities library, SMALLC88.LIB, the MS-DOS linker, LINK.EXE, the

program EXE2BIN.EXE and the MS-DOS assembler, MASM.EXE. The disk will be nearly

full when you're done.

II.     Compiling a Program -- Menu Method

We  start   by booting up disk ALPHA. The text editor and the compiler are

accessible.  Mount a second disk (BETA) -- which need not be bootable -- on B:.

Using the text editor, build the source-code file for your program. The name of

this  file  should  have a suffix of ".C". While this suffix name is not always

mandatory,  the  suffix will indicate what the file is for and should always be

used. Let's say that the filename is "SAMPLE.C".

At this point, we have the following situation:

Drive A:                                Drive B:

Disk ALPHA                              Disk BETA

<MS-DOS O/S>                            SAMPLE.C

<text editor>

C88.COM

\C\INCLUDE\STDIO.H

MASM.EXE

EXE2BIN.EXE

CMAKE.BAT

SMALLC88.LIB

CRUN.OBJ

Now  we  may  compile SAMPLE.C. There are two ways to invoke the compiler.

One  method is to type all of the necessary control information  on the command

line.  The other method is best described as twenty-questions mode. We will try

this  method  first  because  the  questions  and  answers  will  more  clearly

illustrate  the  purpose  of  the  command  line argument method. To invoke the

menu-driven mode just type the compiler name and hit Return:

-2-

C88

The  compiler  will  identify  itself and then begin asking questions. The

first question is:

Do you wish to annotate the assembly-code?

The small-c compiler converts the input source-code file (in the C language) to

assembly-code  compatible  to the MS-DOS assembler. If you answer "yes" to this

question  the  compiler  will  insert  into  the  generated  assembly-code  (as

comments)  the  corresponding source-code statements. Answering "no" (actually,

"y"   or  "n"  is  enough)  will,  as  you  might  imagine,  not  annotate  the

assembly-code.  Usually,  answer  "yes" if you expect to have to "touch up" the

generated code or if you want to have a report for debugging. Remember, though,

that  comments  make  the  assembly-code file bigger; you might not have enough

space on your disk for a really large program with comments.

The next question is a little obscure:

Do you wish the globals to be defined?

The  small-c  compiler  has  two  mechanisms  for  implementing  the C language

"extern"  capability.  One  method  is  to  use  the  "extern"  keyword  with a

declaration,  i.e.:  "extern int a;". The other method is to not define globals

when  compiling.  The  consequence  of  saying "no" to the above question is to

effectively  prefix  the  declaration  of ALL global variables (See Kernighan &

Ritchie  for  the  definition) with "extern". Usually, you will answer "yes" to

this question.

The third question,

Starting number for labels?

is almost always answered with carriage return. However, if you plan to compile

a  very  large  program  (e.g.:  the compiler itself!), it is possible that the

program  would  overflow  the internal symbol table or loop nesting arrays that

the  compiler  uses  while processing the source file. Under these  conditions,

you  would have to split the program into several pieces (C88 is in twelve) and

use "INCLUDE" statements to combine the code fragments at assembly-time.

If  the  code  being  compiled is a fragment of a larger program, you must

give  a starting label number (a decimal integer between 1 and 65535) such that

all of the labels which the compiler creates ("@####") are unique.

Please note that the above paragraphs do not apply when you intend to also

separately  ASSEMBLE (i.e.: use LINK.EXE to combine object files) code; in that

case,  labels  may always begin at the default, since labels are private to the

currently-assembled module.

The next question is:

Output filename?

Normally, you would answer with the assembly-code filename (in our example, the

-3-

answer  would  be  "B:SAMPLE.ASM") you want to create. Alternatively, answer by

hitting a carriage return. This will cause the generated assembly-code to go to

CON  where  you  can  watch  it.  Hint:  it  is  also  possible to answer "PRN"

here. If you do, the compiler will be able to output the assembly code directly

to the printer.

Finally, the compiler will ask:

Input filename?

To  this  question,  answer with the name of the file to be compiled.  You MUST

give  the ".C" suffix; the compiler won't assume it. In our example, the proper

response would be "B:SAMPLE.C".

This  last question will be repeated when the currently-specified file has

been  completely processed. This allows the compiler to compile a program whose

source  file is in several sections but is to be compiled as a single unit. One

effective  use of this capability is to have a bunch of  small C-language files

that  are  each one a simple function or group of functions which is frequently

used.  You  would then first give the name of the main function's file and then

the names, as necessary, of all of your "library" function files.

So, for our example, the compiler would be invoked as:

C88 <CR>

(identification)

Do you wish to annotate the assembly code? Y <CR>

Do you wish the globals to be defined? Y <CR>

Starting number for labels? <CR>

Output filename? B:SAMPLE.ASM <CR>

Input filename? B:SAMPLE.C <CR>

Input filename? <CR>

(where  <CR>  represents  carriage return). After the compiler exits to MS-DOS,

the contents of the disks will be:

Drive A:                                Drive B:

Disk ALPHA                              Disk BETA

<MS-DOS O/S>                            SAMPLE.C

<text editor>                           SAMPLE.ASM

C88.COM

\C\INCLUDE\STDIO.H

MASM.EXE

EXE2BIN.EXE

CMAKE.BAT

SMALLC88.LIB

CRUN.OBJ

This completes the compilation phase of operations.

-4-

If  you  have  followed this method, the next step is described in section

IV,  on  page  5.  However,  a  simpler  way  to use the compiler for "vanilla"

compilation is given in section III, below.

III.    Compiling a Program -- Command Line Method

Again  we  boot disk ALPHA and mount disk BETA in B:. As before, we use a

text  editor  to  construct  SAMPLE.C on drive B: which will be the source-code

file to be compiled.

At this point, we again have the following situation:

Drive A:                                Drive B:

Disk ALPHA                              Disk BETA

<MS-DOS O/S>                            SAMPLE.C

<text editor>

C88.COM

\C\INCLUDE\STDIO.H

MASM.EXE

EXE2BIN.EXE

CMAKE.BAT

SMALLC88.LIB

CRUN.OBJ

Now we may compile SAMPLE.C. The command-line method requires that we give

answers  to the questions that the compiler is asking on the command line. This

is done with "switches", like you would use with MASM. The compiler has default

answers  for its questions; if you don't specify a switch then the default will

be  used.  You MUST provide the name of the input file (and only one input file

may  be specified); all other arguments are optional. The following table gives

the default and switch-settable answer to the compiler's questions:

Question            Default Answer      Switch      Switch setting

----------------------    --------------      ------      --------------

Annotate assembly-code          no              /A             yes

Define globals                  yes             /G              no

Starting label number          zero           /L=####         ####

Output filename            <infile>.ASM       /O=<name>       <name>

The  output filename setting needs a little explaination: if you do not specify

the  switch,  C88  will  strip the ".C" suffix from the input file name (if the

suffix  is given -- it's optional when command line method is used) and replace

it with ".ASM". In our example, "B:SAMPLE.C" would imply "B:SAMPLE.ASM". If you

specify the /O switch, you must give the drive prefix and ".ASM" suffix.

Compiling in our example by this method would be:

C88 /A B:SAMPLE

which  is  much simpler.  Remember, though, that the command line method is not

as flexible; you cannot specify multiple input files.

-5-

This method will yeild the same disk contents as the other, namely:

Drive A:                                Drive B:

Disk ALPHA                              Disk BETA

<MS-DOS O/S>                            SAMPLE.C

<text editor>                           SAMPLE.ASM

C88.COM

\C\INCLUDE\STDIO.H

MASM.EXE

EXE2BIN.EXE

CMAKE.BAT

SMALLC88.LIB

CRUN.OBJ

IV.     Assembling the Compiled Code

Once  the   C-language  source  code has been compiled without errors -- I

should probably mention that the compiler will say "Compilation failed." before

it returns to MS-DOS if there was an error and leaves an error message imbedded

in the ASM file (another reason for outputting to CON for the first try) -- you

will have to assemble the generated assembly-code file.

The current situation with respect to the contents of the disks is:

Drive A:                                Drive B:

Disk ALPHA                              Disk BETA

<MS-DOS O/S>                            SAMPLE.C

<text editor>                           SAMPLE.ASM

C88.COM

\C\INCLUDE\STDIO.H

MASM.EXE

LINK.EXE

EXE2BIN.EXE

CMAKE.BAT

SMALLC88.LIB

CRUN.OBJ

Using our example file, SAMPLE, run the CMAKE.BAT script by typing:

CMAKE B:SAMPLE

The result will be:

Drive A:                                Drive B:

Disk ALPHA                              Disk BETA

<MS-DOS O/S>                            SAMPLE.C

<text editor>                           SAMPLE.ASM

-6-

\C\INCLUDE\STDIO.H

MASM.EXE

LINK.EXE                                SAMPLE.OBJ

EXE2BIN.EXE                             SAMPLE.EXE

CMAKE.BAT                               SAMPLE.COM

SMALLC88.LIB                            SAMPLE.MAP

CRUN.OBJ

You  may  now copy SAMPLE.COM to another disk for debug and use. The files

SAMPLE.OBJ,  SAMPLE.EXE  and  SAMPLE.MAP are unnecessary (if everything works!)

and may be DELeted.

V.      Final Remarks

I   realize  that  the  foregoing  seems terribly complex when compared to

using  ZBASIC, say, but the advantages of having compiled code usually outweigh

the  disadvantages  of  having  a  more  complex  and  less interactive program

generation  process.  If  you  expect  to  use  a  program twenty times and the

compiled  code  will  take eight minutes where interpreted code would take ten,

then  if  it takes about thirty minutes to compile and assemble the program you

have actually SAVED time!

Also, since the small-c language supports command line argument capture by

the compiled code -- not just the compiler -- invoking your program may be made

simpler.  Imagine how much more difficult the MS-DOS "COPY" command would be if

it  had  to ask you for "Source file?" and "Destination file?" instead of being

able get the information out of the command line. With BASIC, all commands must

ask  questions;  with  small-c,  the PROGRAMMER can decide whether questions or

command line arguments is more appropriate.

You  may  want to try the above procedure with the sample programs included

with  the  compiler.  Programs  BUG1  and  BUG2  are variations on PARROT which

contain  trivial  mistakes.  BUG1  will not compile while BUG2 will compile but

will  not assemble because of an undefined function. Be sure to try these since

they  will  help  you become familiar with small-c's error handling scheme. The

other  sample  programs  will  compile  and  assemble  to  functional code; see

"EXAMPLES.DOC" for a complete description of what they do.

Please  remember  that small-c is NOT identical to full UNIX C. Be sure to

read the document "SMALLC.DOC" which describes the differences  between small-c

and  UNIX  C as described in "The C Programming Language" by Brian W. Kernighan

and  Dennis M. Ritchie.  This book is the only published standard for C; nobody

who wants to write C code should be without it.

Another book I would recommend is "The Small-C Handbook" by James Hendrix.

While  his  implementation of small-c differs from this one (it's more advanced

in  some  respects   and  just  different  in  others),  it  does a good job of

describing the small-c dialect.

Finally, we all owe a heartfelt vote of thanks to Ron Cain who started the

whole  small-c ball rolling with his articles in Doctor Dobb's Journal in 1980.

The compiler you're looking at (and nearly all other microcomputer C compilers)

is  derived  from  his  which  he  very generously placed in the public domain.

It started life on my H-8 system and was "ported" to my H-110. The HDOS version

is also available through HUG.